# Die change on Press 14 (sample SOP)

1. Tell the crew that Press 14 is going down for a die change.

2. Stop the press and lock out the main energy supply.

3. Try to start the press. It must not move.

4. Set the safety blocks under the ram.

5. Unclamp and remove the old die.

6. Clean the bolster plate.

7. Seat the new die and tighten the clamps to 85 Nm.

8. Remove the safety blocks and the locks, then run three dry cycles.